|  | Stock | Street Prepared | Modified |
| --- | --- | --- | --- |
| Classification Intent | Stock engines, suspensions, tires and bodies; may be updated to the Series V level. | Upgraded to improve the appearance, drivability, performance, handling and safety without modifying the basic body structure and suspension geometry; may be updated to the Series V level. | Includes competition cars as well as highly modified Alpines. |
| General | No performance options. Must be legally registered and licensed to drive on public streets. | Performance options allowed. Must be legally registered and licensed to drive on public streets. | Allows substantial modification of the body structure, powertrain and suspensions. |
| Body | Unmodified except for cutting of front valence for tire clearance. | Unmodified except for cutting of front valence and rolling back of inner lips of rear fenders for tire clearance. | Full tube construction with composite skin allowed. Body must have the appearance and silhouette of an Alpine. Fender flares allowed. |
| Engine | Must be stock Alpine four-cylinder engine of 1492, 1592 or 1725 cc. displacement with stock Alpine intake and exhaust systems. | Must be stock Alpine four-cylinder engine of 1492, 1592 or 1725 cc. displacement. Webber carburetors allowed. | Any piston engine of no more than 6 cylinders or rotary engine swap is allowed. Any induction system is allowed. |
| Transmission | Alpine four-speed manual, four-speed manual with overdrive or automatic may be used. | Alpine four-speed manual, four-speed manual with overdrive or automatic may be used. | Any manual or automatic transmission may be used. |
| Rear Axle | Limited to stock 3.89:1 or 4.22:1 ratio and open differential. | Stock axles with a 3.89:1 or 4.22:1 ratio and an open, limited or locked differential may be used. | Any solid or independent rear axle is permitted. |

|  | Stock | Street Prepared | Modified |
| --- | --- | --- | --- |
| Wheels | Maximum wheel size is limited to stock 4 1/2" X 13". May be of any material. | Maximum wheel width of 7 inches is allowed. Wheels of any diameter may be used. May be of any material. | Any material and size is allowed. |
| Tires | Must have DOT approved tires with visible tread. DOT approved race tires may not be used. | Must have DOT approved tires. DOT approved race tires may be used. Only modifications allowed are having treads shaved or trued. | Any size, type or construction is allowed. Racing tires are allowed. |
| Front Suspension | Limited to stock suspension, attachment points, geometry, ride height and alignment. | Any springs, anti-roll bars, pivot bushings and shock absorbers may be used attached at the stock mounting points. Ride height may be altered. | Any front suspension is permitted. |
| Rear Suspension | Springs, attachment points and geometry must remain stock. | Leaf springs attached at the stock mounting points are permitted. Spring rates, shock absorbers and ride heights may be altered. Rear anti-sway bars are not allowed. | Any rear suspension is permitted. |
| Steering | Recirculating ball and stock geometry must be used. | Recirculating ball and stock geometry must be used. | There are no restrictions. |

* Please see the full rules for complete classification requirements