|  | - Stock | - Street Prepared | - Modified |
| --- | --- | --- | --- |
| Classification Intent | Stock engines, suspensions, tires and bodies; may be updated to the Mark II level. | Upgraded to improve the appearance, drivability, performance, handling and safety without modifying the basic body structure and suspension geometry; may be updated to the Mark II level. | Includes competition cars as well as highly modified Tigers. |
| General | Minimal performance options. Must be legally registered and licensed to drive on public streets. | LAT and other performance options allowed. Must be legally registered and licensed to drive on public streets. | Allows substantial modification of the body structure, power train and suspensions. |
| Body | Unmodified except for cutting of front valence for tire clearance. | Unmodified except for cutting of front valence and rolling back of inner lips of rear fenders for tire clearance. | Full tube construction with composite skin allowed. Body must have the appearance and silhouette of a Tiger. Fender flares allowed. |
| Engine | Must be stock Ford V8 of 260 or 289 cubic inch displacement with two or four barrel carburetor and stock exhaust manifold. | Must be Ford V8 of 260, 289, or 302 cubic inch displacement in the stock location. Aluminum blocks not allowed. Induction systems are limited to single or multiple carburetors or fuel injection. | Based on the Ford small block V8, iron or aluminum block, in the stock location with no limit to the bore and stroke. Any induction system is allowed. |
| Transmission | Limited to Borg-Warner T-10 or Ford Top Loader four-speed with either a “narrow or “wide” ratio. | Automatic transmissions or alternate manual transmissions (e.g. T-5 5 speed) are allowed. | Any manual or automatic transmission may be used. |
| Rear Axle | Limited to stock Salisbury axle with a 2.88:1 ratio and open differential. | Stock Salisbury axle must be used of any available ratio with open, limited slip or locked differential. | Any solid or independent rear axle is permitted. |
| Wheels | Maximum wheel size is 5 1/2" X 13". LAT70’s, or equivalent, are allowed. May be of any material. | Maximum wheel width of 7 inches is allowed. Wheels of any diameter may be used. May be of any material. | Any material and size is allowed. |

|  | - Stock | - Street Prepared | - Modified |
| --- | --- | --- | --- |
| Tires | Must have DOT approved tires with visible tread. DOT approved race tires may not be used. | Must have DOT approved tires. DOT approved race tires may be used. Only modifications allowed are having treads shaved or trued. | Any size, type or construction is allowed. Racing tires are allowed. |
| Front Suspension | Limited to stock suspension, attachment points, geometry, ride height and alignment. | Any springs, anti-roll bars, pivot bushings and shock absorbers may be used attached at the stock mounting points. Ride height may be altered. | Any front suspension is permitted. |
| Rear Suspension | Springs, attachment points and geometry must remain stock. | Leaf springs, panhard rod, and LAT or equivalent rear axle traction bars (attached at the stock LAT mounting points) are permitted. Spring rates, shock absorbers and ride heights may be altered. Rear anti-sway bars are not allowed. | Any rear suspension is permitted. |
| Steering | Rack and pinion, steering arms and resulting geometry must be stock. | Rack and pinion, steering arms and resulting geometry must be stock. | There are no restrictions. |

* Please see the full rules for complete classification requirements